TCP/IP Protocol Suite Model

Server-Client Model

- Centralized Computing Method

IBM Mainframe: 1965

UNIX OS			:	1969 (AT&T Bell) –

- Distributed Computing Method:

Microsoft, Linux, IBM

DoD US Army: 1970: Published In Public Interest in 1978.

Microsoft, Linux, Unix, IBM, Cisco, HP…………

Protocol: Set of Rules for Communication.

- Data

Standard: common set of rules to define the entity.

- Devices

- Speed Of Communication, Media, Connectors, Pin Outs for Connectors, Shape & Size of the Connector (RJ-45, RJ-11, SC, ST Connector), Material of Connectors.

4 Layer Structured Model (5 Layer)

Application:

HTTP (80), HTTPS (443),

FTP (20/21), FTPS, SFTP,

TelNet (23), SSH (22),

SMTP (25), POP3 (110), NNTP (119),

SNMP (161)-UDP – Cisco CDP

DNS (53) – UDP&TCP, DHCP (67&68) - UDP, LDAP (389) –UDP&TCP.

Client Side Port # - Randomly Selected by Client System (1023+ to 65,535)

Transport:  TCP, UDP & SCTP

TCP: Transmission Control Protocol

: Connection Oriented:

- Connection Establishment – Negotiation Phase – 3 Way Handshake

- Connection Maintenance – Windowing (By changing the Window Size time to time.

Data Transfer

- Connection Termination – 2 way Handshake (Finalization Flags)

: Reliable (Sequence Number(s) and Acknowledge Number(s)

- Error Detection

- Error Correction (By resending the Segments)

UDP: User Datagram Protocol

: Connection-Less

: Unreliable

: Faster

SCTP (Stream Control Transmission Protocol)

Live Stream Data: Audio/Video

Internet: IP (IPv4 & IPv6), ICMP, IGMP, ARP, RARP, IPSec.

IP Address: (Internet Protocol) – Internet, Intranet, ExtraNet.

- Logical Address

- Software Based Address (API – Configure, Reconfigurable)

- 32 Bit Address – 232= 4.3 Billion

- IANA (Internet Assigned Numbers Authority)

- Its combination of Network ID + Host ID

- 4 Octets (Octet=8 Bits), Separated by “.”

- Represented by Decimal Number.

- W.		X.		Y.		Z 	(Cisco – A. B. C. D)

- 8 Bits.	8 Bits.	8Bits.	8Bits

- Classes – 5 – A, B, C, D & E

100=> 	1			0			0

102=100	101=10		100=1  100+0+0=> 100

27		26		25		24		23		22		21		20

128		64		32		16		8		4		2		1

0		0		0		0		0		0		0		0  0

1		1		1		1		1		1		1		1  255

Class A:		W.				X.				Y.				Z

| N/w ID-.	|-----------------Host ID = 224=1,67,77,216|

00000000  0   (0.0.0.0 It represents All Possible N/Ws or 										Hosts.

00000001  1 	(1.0.0.0)

01111111  127 (127.0.0.0)  Reserved for Loopback

- 127.0.0.1  Localhost

Class A – 1-126  10.0.0.0

Class B:		W.				X.				Y.				Z

| N/w ID= 16 Bits--------.-Host ID = 216=65,536|

10000000  128

10111111   191

Class B – 128 -191  150.0.0.0

Class C:		W.				X.				Y.				Z

| N/w ID= 24 Bits-----------------------.-Host ID =28=256|

11000000  192

11011111   223

Class C – 192 -223  192.168.10.10

Class D:		W.				X.				Y.				Z

| ------------------------N/w ID= 32 Bits------------------|

11100000  224

11101111   239

Class D – 224 -239  230.0.0.0

Class D – Reserved for Multicast Communication.

Class E:		W.				X.				Y.				Z

| ------------------------N/w ID= 32 Bits------------------|

11110000  240

11111111  255

Class E – 240 -255  255.255.255.255

Class D – Reserved for R & D. (Broadcast ID)

Unicast: One-to-One Communication – Class A,B,C

Multicast: One-to-Many (But Not All) – Group Communication – IGMP – Class D

Broadcast: One-to-All Communication – Class E

Anycast:

IETF: 1995 – Private IP & Public IP

Private IP: 	Intranet

The IP Packet having Private IP can’t communicate over 					Internet.

NAT (Network Address Translation):

Private IP Address:

Class A: 10.0.0.0 – 1 N/w Reserved

Class B: 172.16.0.0 – 172.31.0.0 – 16 N/w Reserved

Class C: 192.168.0.0 – 256 N/w Reserved

APIPA (Automatic Private IP Address): 169.254.0.0: Link Local

192.168.10.1:

192.168.10. – Network ID + .1 Host ID

SubNetmask (Netmask) Address:

- Logical Address

- Software Based Address (API – Configure, Reconfigurable)

- 32 Bit Address – 232= 4.3 Billion

- 4 Octets (Octet=8 Bits), Separated by “.”

- Represented by Decimal Number.

- W.		X.		Y.		Z 	(Cisco – A. B. C. D)

- 8 Bits.	8 Bits.	8Bits.	8Bits

- Its combination of Contiguous 1s (Network ID) + Remaining Contiguous 0s (Host ID)

128		64		32		16		8		4		2		1

- 0		0		0		0		0		0		0		0  0

- 1		1		1		1		1		1		1		1  255

192.168.10.1

255.255.255.0

192.				168.			10.			1

11000000.10101000.00001010.00000001

11111111.11111111.11111111.00000000

11000000.10101000.00001010.00000000

192.168.10.0

255.255.255.0

To extract the Network ID from the IP Address (Network ID + Host ID).

Subnet Mask					Pre-fix Length

Class A: 255.0.0.0				/8

Class B: 255.255.0.0				/16

Class C: 255.255.255.0			/24

Class D: 255.255.255.255		/32

Classful Network:	172.16.0.0/16

Classless Network: 	172.16.0.0/24

IPv6 (Dec 1998)

- Logical Address

- Software Based Address (API – Configure, Reconfigurable)

- 128 Bit Address – 2128

- 8 Batches (Batch=16 Bits), Separated by “:”

- Represented by Hex Decimal Number. (0-9, A&F)

- 2022:0000:0000:0000:0001:0023:0456:789A/64

- No Classes

- 64 Bits (Network ID) + 64 Bits (Host/Interface ID)

- Zero Shrinking Rules:

- 1) All Leading Zeros can be eliminated.

- 2022:0000:0000:0000:0001:0023:0456:789A/64

- 2022:0:0:0:1:23:456:789A/64

- 2) Contiguous Zero Fields can be replaced with “::” (But Only Once in an Address)

- 2022:0:0:0:1:23:456:789A/64

- 2022::1:23:456:789A

| 23=8 | 22=4 | 21=2 | 20=1 | Value |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 1 | 1 |
| 0 | 0 | 1 | 0 | 2 |
| 0 | 0 | 1 | 1 | 3 |
| 0 | 1 | 0 | 0 | 4 |
| 0 | 1 | 0 | 1 | 5 |
| 0 | 1 | 1 | 0 | 6 |
| 0 | 1 | 1 | 1 | 7 |
| 1 | 0 | 0 | 0 | 8 |
| 1 | 0 | 0 | 1 | 9 |
| 1 | 0 | 1 | 0 | A |
| 1 | 0 | 1 | 1 | B |
| 1 | 1 | 0 | 0 | C |
| 1 | 1 | 0 | 1 | D |
| 1 | 1 | 1 | 0 | E |
| 1 | 1 | 1 | 1 | F |

Unicast:	1) Unique Local Unicast Address (Private):

F 		C	0	0::/7

11111100 = FC

11111101 = FD

- Unique Global Unicast Address (Public):

2000::/3

2		0		0		0::/3

0010 2

0011 3

- Link-Local Unicast Address: FE80::/10

F			E		8		0

111111101000  FE8

111111101001  FE9

111111101010  FEA

111111101011  FEB

Multicast Address: FF00::/8

F		F

11111111 FF